Copyright 2018 by The American Institute of Architects (AIA)

Exclusively published and distributed by AVITRU, LLC for the AIA

This Product MasterSpec Section is licensed by AVITRU, LLC to Oatey ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Avitru or The American Institute of Architects (AIA). Neither AIA nor Avitru are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Oatey, 20600 Emerald Pkwy, Cleveland, OH 44135; Phone: (216) 267-7100; Website: www.oatey.com; Email: info@oatey.com.

For information about MasterSpec, contact Avitru at (800) 424-5080 or visit www.avitru.com.

SECTION 102800 - TOILET, BATH, AND LAUNDRY ACCESSORIES

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Public-use washroom accessories.

          - Public-use shower room accessories.

          - Private-use bathroom accessories.

          - Healthcare accessories.

          - Warm-air dryers.

          - Childcare accessories.

          - Underlavatory guards.

          - Custodial accessories.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 088300 "Mirrors" for frameless mirrors.

          - Section 093013 "Ceramic Tiling" for ceramic toilet and bath accessories.

          - Section 102813.63 "Detention Toilet Accessories" for accessories designed for installation in detention facilities.

      - COORDINATION

        - Coordinate accessory locations with other work to prevent interference with clearances required for access by people with disabilities, and for proper installation, adjustment, operation, cleaning, and servicing of accessories.

        - Deliver inserts and anchoring devices set into concrete or masonry as required to prevent delaying the Work.

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.

          - Include anchoring and mounting requirements, including requirements for cutouts in other work and substrate preparation.

Retain subparagraph below if units are required that connect to building electrical system.

          - Include electrical characteristics.

If retaining "Samples" Paragraph below, consider revising to limit sample requirements to specific items.

        - Samples: Full size, for each exposed product and for each finish specified.

Retain subparagraph below if applicable.

          - Approved full-size Samples will be returned and may be used in the Work.

        - Product Schedule: Indicating types, quantities, sizes, and installation locations by room of each accessory required.

          - Identify locations using room designations indicated.

Retain subparagraph below if product designations are indicated.

          - Identify accessories using designations indicated.

      - INFORMATIONAL SUBMITTALS

        - Sample Warranty: For manufacturer's special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For accessories to include in maintenance manuals.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Special Warranty for Mirrors: Manufacturer agrees to repair or replace mirrors that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, visible silver spoilage defects.

Not all manufacturers offer a special mirror warranty. Verify available warranties and warranty periods.

          - Warranty Period: [15] <Insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Avitru. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - OWNER-FURNISHED MATERIALS

For large commercial projects, campus-type projects with multiple buildings, or similar situations, Owner may have separate service contracts for furnishing various accessories and corresponding supplies, such as paper towels along with towel dispensers. In some cases such suppliers will install these accessories, which can create coordination problems if not properly located in plan or in height. If Owner will furnish accessories to Contractor for installation, retain this article and insert requirements in "Owner-Furnished Materials" Paragraph below.

        - Owner-Furnished Materials: <Insert products>.

      - PERFORMANCE REQUIREMENTS

Retain this article if accessories that connect to building electrical systems, such as warm-air dryers, are required.

        - Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

      - PUBLIC-USE WASHROOM ACCESSORIES

        - Source Limitations: Obtain public-use washroom accessories from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers, and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Toilet Tissue (Roll) Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Roll-in-reserve dispenser with hinged front secured with tumbler lockset] [Single-roll dispenser] [Double-roll dispenser] [Double-roll dispenser with shelf] <Insert description>.

          - Mounting: [Recessed] [Partition mounted, serving two adjacent toilet compartments] [Surface mounted].

Controlled-delivery units cannot be used at accessible toilets.

          - Operation: [Noncontrol delivery with standard spindle] [Noncontrol delivery with theft-resistant spindle] [Spindleless with tension-spring controlled delivery] [Spindleless with tension-spring controlled delivery and self-locking device extending through core that prevents core removal until roll is empty] [Eccentric-shaped, molded-plastic spindle revolves one-half revolution per dispensing operation for controlled delivery; core cannot be removed until roll is empty] <Insert description>.

          - Capacity: Designed for [4-1/2- or 5-inch- (114- or 127-mm-)] [5-inch- (127-mm-)] <Insert dimension> diameter tissue rolls.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Chrome-plated zinc alloy (zamac) or steel] [Satin-finish aluminum bracket with plastic spindle] [ABS plastic, gray] <Insert material and finish>.

Combination toilet tissue dispenser units are available with sanitary-napkin disposal units, seat-cover dispensers, or both.

        - Combination Toilet Tissue Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Combination unit with double-roll toilet tissue dispenser and the following:

            - Removable sanitary-napkin waste receptacle with self-closing, disposal-opening cover.

            - Seat-cover dispenser with minimum capacity of [500] [1000] <Insert number> single or half-fold seat covers.

          - Mounting: [Recessed] [Surface mounted] [Partition mounted, dual access with two tissue rolls per compartment] [Partition mounted, dual access with two tissue rolls per compartment and with one side that mounts flush with partition of accessible compartment].

          - Toilet Tissue Dispenser Capacity: 4-1/2- or 5-inch- (114- or 127-mm-) diameter tissue rolls.

          - Toilet Tissue Dispenser Operation: [Noncontrol delivery with theft-resistant spindles] <Insert description>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Toilet Tissue (Jumbo-Roll) Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [One-roll unit] [Two-roll unit with sliding panel to expose other roll].

          - Mounting: Surface mounted.

          - Capacity: [9- or 10-inch- (228- or 254-mm-)] <Insert dimension(s)> diameter rolls.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

          - Refill Indicator: Pierced slots at front.

        - Paper Towel (Folded) Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Recessed] [Semirecessed] [Deck mounted, recessed] [Surface mounted].

Single-fold towels usually require 6-inch- (152-mm-) deep dispensers. Dispensers 4 inches (102 mm) in depth accommodate C-fold or multifold towels. C-fold towels are the highest quality and softest folded towels; single-fold towels are the least expensive.

          - Minimum Capacity: [400 C-fold or 525 multifold towels] [600 C-fold or 800 multifold towels] [400 single-fold towels] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

Retain "Refill Indicator" Subparagraph below if required.

          - Refill Indicator: Pierced slots at sides or front.

Roll towel dispensers extend farther from the wall than folded towel dispensers.

        - Paper Towel (Roll) Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Lever-actuated mechanism permitting controlled delivery of paper rolls in preset lengths per stroke.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Automatic Paper Towel (Roll) Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - <Insert manufacturer's name>.

          - Description: Automatic motion sensing mechanism with user-adjustable delay and paper towel length; battery powered.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Waste Receptacle <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Open top, recessed] [Self-closing disposal-opening cover, recessed] [Semirecessed] [Surface mounted] [Wall mounted for corner installation] [Freestanding] [Undercounter] <Insert description>.

          - Minimum Capacity: <Insert gal. (L)>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Liner: [Reusable vinyl liner] <Insert liner description>.

          - Lockset: Tumbler type for waste receptacle.

        - Combination Towel (Folded) Dispenser/Waste Receptacle <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Combination unit for dispensing C-fold or multifold towels, with removable waste receptacle.

          - Mounting: [Surface mounted] [Surface mounted with stainless-steel collar] [Recessed] [Recessed with projecting receptacle] [Semirecessed].

First subparagraph below does not apply to surface-mounted units.

            - Designed for nominal [4-inch (100-mm)] [6-inch (150-mm)] wall depth.

          - Minimum Towel-Dispenser Capacity: [600 C-fold or 800 multifold paper towels] <Insert capacity>.

          - Minimum Waste-Receptacle Capacity: [4 gal. (15 L)] [12 gal. (45.4 L)] <Insert value>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Liner: [Reusable, vinyl waste-receptacle liner] <Insert liner description>.

          - Lockset: Tumbler type for towel-dispenser compartment[ and waste receptacle].

        - Combination Towel (Roll) Dispenser/Waste Receptacle <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Combination unit for dispensing preset length of roll paper towels, with removable waste receptacle.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Towel-Dispenser Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Minimum Waste Receptacle Capacity: [8 gal. (30 L)] [12 gal. (45.4 L)] [15 gal. (56.8 L)] <Insert value>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Liner: [Reusable, vinyl waste-receptacle liner] <Insert liner description>.

          - Lockset: Tumbler type for towel dispenser compartment[ and waste receptacle].

        - Multipurpose Soap/Towel Dispenser Unit <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Combination unit for dispensing soap in [liquid or lotion] [lather] form and folded towels.

          - Mounting: [Recessed, designed for nominal 4-inch (100-mm) wall depth] [Surface mounted with stainless-steel collar].

          - Minimum Soap-Dispenser Capacity: [80 oz. (2.36 L)] <Insert value>.

          - Minimum Towel-Dispenser Capacity: [600 C-fold or 800 multifold towels] [1000 single-fold towels] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish> for unit body and soap valve.

          - Lockset: Tumbler type.

Coordinate requirements for soap dispensers with other Sections if a central soap storage tank is included.

        - Liquid-Soap Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

In "Description" Subparagraph below, "antibacterial" option is for soaps that may be corrosive to soap dispenser parts. Obtain information from Owner about the type of soap used.

          - Description: Designed for dispensing [antibacterial ]soap in [liquid or lotion] [lather] form.

          - Mounting: [Deck mounted on vanity] [Deck mounted on lavatory] [Horizontally oriented, recessed] [Horizontally oriented, surface mounted] [Vertically oriented, surface mounted].

          - Capacity: <Insert oz. (mL)>.

          - Materials: <Insert requirements for valve and reservoir materials and finishes>.

Retain "Lockset" and "Refill Indicator" subparagraphs below if required.

          - Lockset: Tumbler type.

          - Refill Indicator: Window type.

        - Automatic Liquid-Soap Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bradley Corporation.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Sloan Valve Company.

            - Stern Engineering Ltd.

            - <Insert manufacturer's name>.

In "Description" Subparagraph below, "antibacterial" option is for soaps that may be corrosive to soap dispenser parts. Obtain information from Owner about the type of soap used.

          - Description: Automatic dispenser with infrared sensor to detect presence of hands; battery powered; designed for dispensing [antibacterial ]soap in [liquid or lotion] [lather] form.

Some manufacturers may provide wall-mounted automatic units.

          - Mounting: Deck mounted on [vanity] [lavatory].

          - Capacity: <Insert oz. (mL)>.

          - Materials: <Insert requirements for valve and reservoir materials and finishes>.

Retain "Refill Indicator" and "Low Battery Indicator" subparagraphs below if required.

          - Refill Indicator: LED indicator.

          - Low Battery Indicator: LED indicator.

        - Grab Bar <Insert drawing designation>:

For information on Oatey's "Dearborn Grab Bars," see www.oatey.com/126534/Category/Grab-Bars.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Oatey; Dearborn Grab Bars or comparable product by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: Flanges with [concealed] [exposed] fasteners.

          - Material: Stainless steel, 0.05 inch (1.3 mm) thick.

            - Finish: [Peened finish] [Smooth, ASTM A480/A480M No. 4 finish (satin)][ on ends and slip-resistant texture in grip area].

Retain first option in "Outside Diameter" Subparagraph below for medium-duty applications, second option for heavy duty.

          - Outside Diameter: [1-1/4 inches (32 mm)] [1-1/2 inches (38 mm)].

          - Configuration and Length: Straight, [12 inches (305 mm) long] [16 inches (406 mm) long] [18 inches (457 mm) long] [24 inches (610 mm) long] [30 inches (762 mm) long] [32 inches (813 mm) long] [36 inches (914 mm) long] [42 inches (1067 mm) long] [48 inches (1219 mm) long].

        - Vendor <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Type: [Sanitary napkin] [Sanitary napkin and tampon] [Condom].

          - Mounting: [Fully recessed, designed for 4-inch (100-mm) wall depth] [Semirecessed] [Surface mounted].

Capacity varies between manufacturers. For competitive bidding, consider indicating minimum capacities based on Project requirements.

          - Capacity: <Insert description>.

          - Operation: [No coin (free)] [Single coin (25 cents)] [Two coins (50 cents)] <Insert description>.

          - Exposed Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

          - Lockset: Tumbler type with separate lock and key for coin box.

        - Sanitary-Napkin Disposal Unit <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Recessed] [Partition mounted, dual access] [Surface mounted].

          - Door or Cover: Self-closing, disposal-opening cover[ and hinged face panel with tumbler lockset].

          - Receptacle: Removable.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

        - Seat-Cover Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Surface mounted] [Recessed] [Partition mounted, dual access].

          - Minimum Capacity: [250] [500] <Insert number> seat covers.

          - Exposed Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Fold-Down Purse Shelf <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Hinged unit with spring-loaded shelf that automatically returns to vertical position.

          - Nominal Size: [15 inches (381 mm) long by 5-1/2 inches (140 mm) wide] <Insert dimensions>.

          - Material and Finish: [Chrome-plated, cast-zinc alloy (zamac) with stippled finish on tray or stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Chrome-plated, cast-zinc alloy (zamac) with stippled finish on tray and bright chrome finish on edges] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)].

        - Mirror Unit <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

In "Frame" Subparagraph below, options for tilted and adjustable tilting mirrors are for use by people with disabilities. Adjustable tilting mirrors are prone to vandalism. In lieu of tilted mirrors, standard flat mirrors can be mounted at heights to accommodate users in wheelchairs.

          - Frame: [Stainless-steel angle, 0.05 inch (1.3 mm) thick] [Stainless-steel channel] [Stainless steel, fixed tilt] [Stainless steel, adjustable tilt].

            - Corners: [Manufacturer's standard] [Mitered and mechanically interlocked] [Welded and ground smooth].

Retain "Integral Shelf" Subparagraph below if required.

          - Integral Shelf: 5 inches (127 mm) deep.

          - Hangers: Produce rigid, tamper- and theft-resistant installation, using method indicated below.

Delete first or second subparagraph below, or retain both and indicate as Contractor's option.

            - One-piece, galvanized-steel, wall-hanger device with spring-action locking mechanism to hold mirror unit in position with no exposed screws or bolts.

Delete first subparagraph below if tilting mirrors are required.

            - Wall bracket of galvanized steel, equipped with concealed locking devices requiring a special tool to remove.

          - Size: [As indicated on Drawings] <Insert dimensions>.

        - Coat Hook <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Double] [Single]-prong unit.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

      - PUBLIC-USE SHOWER ROOM ACCESSORIES

        - Source Limitations: Obtain public-use shower room accessories from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Shower Curtain Rod <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

Retain first option in "Description" Subparagraph below for normal-duty applications, second option for heavy duty.

          - Description: [1-inch (25.4-mm) OD; fabricated from nominal 0.0375-inch- (0.95-mm-) thick stainless steel] [1-1/4-inch (32-mm) OD; fabricated from nominal 0.05-inch- (1.3-mm-) thick stainless steel].

          - Mounting Flanges: [Stainless-steel flanges designed for exposed fasteners] <Insert requirements>.

          - Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert finish>.

        - Shower Curtain <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

Curtains that are 12 inches (305 mm) wider than opening are recommended for shower openings 48 inches (1219 mm) wide and wider.

          - Size: Minimum [6 inches (152 mm)] [12 inches (305 mm)] wider than opening by 72 inches (1828 mm) high.

          - Material: [Vinyl, minimum 0.006 inch (0.15 mm) thick, opaque, matte] [Duck, minimum 8 oz. (227 g), white, 100 percent cotton] [Nylon-reinforced vinyl, minimum 10 oz. (284 g) or 0.008-inch- (0.2-mm-) thick vinyl, with integral antibacterial agent] <Insert material>.

Antibacterial shower curtains are available in green.

          - Color: [White] [Green] [As selected from manufacturer's full range] <Insert color>.

          - Grommets: Corrosion resistant at minimum 6 inches (152 mm) o.c. through top hem.

          - Shower Curtain Hooks: Chrome-plated or stainless-steel, spring wire curtain hooks with snap fasteners, sized to accommodate specified curtain rod. Provide one hook per curtain grommet.

        - Folding Shower Seat <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Configuration: [L-shaped seat, designed for wheelchair access] [Rectangular seat] [Triangular, corner-type seat] [Stainless-steel seat designed to fold into recessed-mounted, stainless-steel wall box].

          - Seat: [Phenolic or polymeric composite of slat-type or one-piece construction in color as selected by Architect] [White vinyl padded seat] [Stainless steel, ASTM A480/A480M No. 4 finish (satin); 0.05-inch (1.3-mm) minimum nominal thickness; with single-piece, pan-type construction and edge seams welded and ground smooth] <Insert description>.

          - Mounting Mechanism: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert description>.

          - Dimensions: <Insert dimensions>.

        - Soap Dish <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [With] [Without] washcloth bar.

          - Mounting: [Recessed] [Surface mounted].

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

        - Robe Hook <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Double] [Single]-prong unit.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

      - PRIVATE-USE BATHROOM ACCESSORIES

        - Source Limitations: Obtain private-use bathroom accessories from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers, and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Toilet Tissue Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Single-roll dispenser] [Double-roll dispenser] [Single-roll dispenser with hood] [Double-roll dispenser with hood] <Insert description>.

          - Mounting: [Recessed] [Surface mounted].

          - Capacity: Designed for 4-1/2- or 5-inch- (114- or 127-mm-) diameter tissue rolls.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Shower Curtain Rod <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Outside Diameter: [1 inch (25.4 mm)] [1-1/4 inches (32 mm)].

          - Mounting: Flanges with [exposed] [concealed] fasteners.

          - Rod Material and Finish: [Solid brass, polished] [Polished chrome-plated brass] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

          - Flange Material and Finish: [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

Retain "Accessories" Subparagraph below if required.

          - Accessories: Integral chrome-plated brass glide hooks.

        - Soap Dish <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: <Insert description>.

          - Mounting: [Recessed] [Surface mounted].

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Medicine Cabinet <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Recessed, for nominal 4-inch (100-mm) wall depth] [Surface mounted].

          - Size: [18 by 24 inches (460 by 610 mm)] <Insert dimensions>.

          - Door: [Framed mirror door concealing storage cabinet equipped with continuous hinge and spring-buffered, rod-type stop and magnetic door catch] <Insert description>.

          - Shelves: [Three, adjustable] <Insert requirements>.

          - Material and Finish:

            - Cabinet: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Steel with corrosion-resistant finish].

            - Mirror Frame: <Insert material and finish>.

            - Door: <Insert material and finish>.

            - Hinge: <Insert material and finish>.

            - Shelves: <Insert material and finish>.

        - Facial Tissue Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Wall mounted, recessed] [Surface mounted].

          - Depth: [2-5/8 inches (67 mm)] [4 inches (102 mm)] <Insert dimension>.

          - Material and Finish:

            - Dispenser Face: [Polished chrome-plated brass] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

            - Cabinet: [Steel with corrosion-resistant finish] <Insert material and finish>.

        - Robe Hook <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Double] [Single]-prong unit.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Toothbrush and Tumbler Holder <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: <Insert description>.

          - Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Towel Bar <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [3/4-inch- (19-mm-) square tube with rectangular end brackets] [3/4-inch- (19-mm-) round tube with circular end brackets] <Insert description>.

          - Mounting: Flanges with [concealed] [exposed] fasteners.

          - Length: [18 inches (457 mm)] [24 inches (610 mm)] [30 inches (762 mm)] <Insert dimension>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] [Polished aluminum] <Insert material and finish>.

        - Towel Pin <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Projecting minimum of [3 inches (75 mm)] [5 inches (127 mm)] from wall surface.

          - Material and Finish: [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Towel Ring <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Pin projecting approximately 2-1/2 inches (63 mm) from wall with [square] [circular] [oval] [trapezoidal] ring.

          - Pin Material and Finish: [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

          - Ring Material and Finish: [Matching pin] [Clear plastic].

        - Towel Shelf <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Surface-mounted, guest-towel shelf with four [3/8-inch- (9-mm-) diameter] [5/16-inch- (8-mm-) square] stainless-steel tubes mounted in support arms.

Retain "Towel Bar" Subparagraph below if required; if retaining, revise to suit Project.

            - Towel Bar: [1/4-inch- (6-mm-) diameter] [5/16-inch- (8-mm-) square] stainless-steel towel bar below shelf.

          - Length: [18 inches (457 mm)] [24 inches (610 mm)] <Insert dimension>.

          - Material and Finish: [Polished brass-plated, stainless steel] [Polished chrome-plated, stainless-steel] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)] <Insert material and finish>.

        - Towel Rack <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - Basco, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Taymore Industries USA Inc.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Surface-mounted, guest-towel unit with approximately 1/4-inch- (6-mm-) diameter wire rings welded to upright wire bracket] <Insert description>.

          - Capacity: [Two] [Three] [Four] <Insert number> sets of bath towels, hand towels, and washcloths.

          - Nominal Height: [11 inches (279 mm)] [17 inches (432 mm)] [21 inches (533 mm)] <Insert dimension>.

          - Material and Finish: [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated zinc alloy (zamac)] <Insert material and finish>.

        - Retractable Clothesline <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Franklin Brass by Liberty Hardware Manufacturing Corporation; a Masco company.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Ginger by Brasstech, Inc.; a Masco company.

            - Seachrome Corporation.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Surface-mounted [rectangular] [circular] housing with minimum 72-inch- (1829-mm-) long, retractable, spring-actuated, synthetic clothesline and remote retention bracket.

          - Material and Finish [Chrome-plated brass] [Stainless steel, ASTM A480/A480M No. 7 finish (polished)].

      - HEALTHCARE ACCESSORIES

        - Source Limitations: Obtain healthcare accessories from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers, and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Specimen Pass-Through Cabinet <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - Carr Corporation.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: With self-closing doors on both sides, lock that prevents doors from both being opened at same time, and removable stainless steel tray.

          - Nominal Wall Opening: [12 by 11-1/4 inches (305 by 285 mm)] <Insert dimensions>, width by height.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

        - Specimen Pass-Through Box <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: With minimum [12-inch- (305-mm-)] <Insert dimension> diameter, turntable removable cylinder that revolves on stainless steel, self-lubricating, ball-bearing plates, and with mechanism to prevent over rotation of cylinder.

          - Nominal Wall Opening: [13-1/4 by 14 inches (335 by 355 mm)] <Insert dimensions>, width by height.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

          - Lockset: Tumbler type.

        - Bedpan and Urinal Cabinet <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: For storing one conventional-size bedpan and one urinal bottle; with door that produces 1/2-inch (13-mm) opening at top and bottom of cabinet to allow air circulation.

          - Mounting: Recessed.

          - Nominal Wall Opening: [13-1/2 by 26-1/2 by 5 inches (340 by 670 by 130 mm)] <Insert dimensions>, width by height by depth.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

        - Bedpan and Urinal Rack <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: For storing one conventional-size bedpan and one urinal bottle.

          - Mounting: Surface mounted.

          - Size: [12 by 27 inches (300 by 685 mm)] <Insert dimensions>, width by height.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

      - WARM-AIR DRYERS

        - Source Limitations: Obtain warm-air dryers from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers, and revise product characteristics accordingly.

Copy paragraph below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Warm-Air Dryer <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Dryer, Inc.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Excel Dryer Inc.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Saniflow Hand Dryer Corporation.

            - Sloan Valve Company.

            - Tubular Specialties Manufacturing, Inc.

            - World Dryer Corporation.

            - <Insert manufacturer's name>.

          - Description: Standard-speed, warm-air hand dryer.

In "Mounting" Subparagraph below, retain last option for surface-mounted units that project from walls by a limited dimension and that comply with the ADA-ABA. Recessed and semirecessed may also comply with the ADA-ABA.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted][, with low-profile design].

          - Operation: [Touch-button] [Electronic-sensor] activated with timed power cut-off switch.

            - Operation Time: [30 to 40] [80] <Insert number(s)> seconds.

          - Cover Material and Finish: [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

          - Electrical Requirements: [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208-240 V, 9-10 A, 1900-2300 W] <Insert electrical requirements>.

        - High-Speed Warm-Air Dryer <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Dryer, Inc.

            - American Specialties, Inc.

            - Bradley Corporation.

            - Excel Dryer Inc.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Saniflow Hand Dryer Corporation.

            - Sloan Valve Company.

            - World Dryer Corporation.

            - <Insert manufacturer's name>.

          - Description: High-speed, warm-air hand dryer for rapid hand drying.

In "Mounting" Subparagraph below, retain last option for surface-mounted units that project from walls by a limited dimension and that comply with the ADA-ABA. Recessed and semirecessed may also comply with the ADA-ABA.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted][, with low-profile design].

          - Operation: Electronic-sensor activated with operation time of [10 to 20] <Insert number(s)> seconds.

          - Cover Material and Finish: [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

          - Electrical Requirements: [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208-240 V, 9-10 A, 1900-2300 W] <Insert electrical requirements>.

        - Multiple Airflow Warm-Air Dryer <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Dyson Inc.

            - Saniflow Hand Dryer Corporation.

            - World Dryer Corporation.

            - <Insert manufacturer's name>.

          - Description: Multiple airflow warm-air hand dryer, using two or more airstreams for rapid hand drying.

In "Mounting" Subparagraph below, retain last option for surface-mounted units that project from walls by a limited dimension and that comply with the ADA-ABA. Recessed and semirecessed may also comply with the ADA-ABA.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted][, with low-profile design].

          - Operation: Electronic-sensor activated with operation time of [10 to 20] <Insert number(s)> seconds.

          - Cover Material and Finish: [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

          - Electrical Requirements: [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208-240 V, 9-10 A, 1900-2300 W] <Insert electrical requirements>.

      - CHILDCARE ACCESSORIES

        - Source Limitations: Obtain childcare accessories from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers, and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Diaper-Changing Station <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Diaper Deck & Company, Inc.

            - Foundations Children's Products.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Koala Kare Products.

            - SafeStrap Company, Inc. (SSC, Inc.).

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Horizontal] [Vertical] unit that opens by folding down from stored position and with child-protection strap.

            - Engineered to support minimum of [250-lb (113-kg)] <Insert value> static load when opened.

          - Mounting: [Surface mounted, with unit projecting not more than 4 inches (100 mm) from wall when closed] [Semirecessed, with unit projecting not more than 1 inch (25 mm) from wall when closed].

          - Operation: By pneumatic shock-absorbing mechanism.

          - Material and Finish: [HDPE in manufacturer's standard color] [HDPE with plastic-laminate insert in color selected by Architect] [Stainless steel, ASTM A480/A480M No. 4 finish (satin), with replaceable insulated polystyrene tray liner and rounded plastic corners] [Stainless steel, ASTM A480/A480M No. 4 finish (satin), exterior shell with rounded plastic corners; HDPE interior in manufacturer's standard color] <Insert requirements>.

          - Liner Dispenser: Built in.

        - Diaper-Pack Vendor <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Diaper Deck & Company, Inc.

            - Foundations Children's Products.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Koala Kare Products.

            - SafeStrap Company, Inc. (SSC, Inc.).

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Surface mounted] [Recessed].

          - Minimum Capacity: [100] <Insert number> diaper packs.

          - Coin Operation: Coin slot preset for [one U.S. dollar, adjustable up in 25-cent increments] <Insert requirements>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] <Insert material and finish>.

        - Child-Protection Seat <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - American Specialties, Inc.

            - Diaper Deck & Company, Inc.

            - Foundations Children's Products.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Koala Kare Products.

            - SafeStrap Company, Inc. (SSC, Inc.).

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Unit that opens by folding down from stored position and with child-protection strap.

            - Engineered to support minimum of [80-lb (36-kg)] [150-lb (68-kg)] <Insert value> static load when opened.

          - Mounting: Surface mounted, with unit projecting not more than [4-1/2 inches (114 mm)] [6 inches (152 mm)] from wall when closed.

          - Material and Finish: [HDPE in manufacturer's standard color] <Insert requirements>.

      - UNDERLAVATORY GUARDS

Delete this article if piping insulation is specified in Section 220719 "Plumbing Piping Insulation" or if architectural woodwork is used to prevent direct contact with piping.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Underlavatory Guard <Insert drawing designation>:

For information on Oatey's "Dearborn Safety Series ADA Compliant Tubular Covers" see www.oatey.com/2376668/Product/Dearborn-Safety-Series-ADA-Compliant-Tubular-Covers.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Oatey; Dearborn Safety Series ADA Compliant Tubular Covers; or comparable product by one of the following:

            - Buckaroos, Inc.

            - Plumberex Specialty Products, Inc.

            - Truebro by IPS Corporation.

            - <Insert manufacturer's name>.

          - Description: Insulating pipe covering for supply and drain piping assemblies that prevents direct contact with and burns from piping; allow service access without removing coverings.

          - Material and Finish: Antimicrobial, molded EVA foam, white.

      - CUSTODIAL ACCESSORIES

        - Source Limitations: Obtain custodial accessories from single source from single manufacturer.

Products offered by manufacturers vary. Verify products offered by manufacturers, and revise product characteristics accordingly.

Copy paragraphs below and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Utility Shelf <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: With exposed edges turned down not less than 1/2 inch (13 mm) and supported by two triangular brackets welded to shelf underside.

          - Size: [16 inches (406 mm) long by 6 inches (152 mm) deep] <Insert dimensions>.

          - Material and Finish: Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel, ASTM A480/A480M No. 4 finish (satin).

        - Mop and Broom Holder <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: [Unit with shelf, hooks, holders, and rod suspended beneath shelf] <Insert description>.

          - Length: [36 inches (914 mm)] <Insert dimension>.

          - Hooks: [Four] <Insert number>.

          - Mop/Broom Holders: [Three] <Insert number>, spring-loaded, rubber hat, cam type.

          - Material and Finish: Stainless steel, ASTM A480/A480M No. 4 finish (satin).

            - Shelf: Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel.

            - Rod: Approximately 1/4-inch- (6-mm-) diameter stainless steel.

If the same paper towel dispensers and soap dispensers that are used in washrooms are also used in custodial areas, delete "Paper Towel (Folded) Dispenser," "Paper Towel (Roll) Dispenser," and "Liquid-Soap Dispenser" paragraphs below.

        - Paper Towel (Folded) Dispenser <Insert drawing designation>:

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

Single-fold towels usually require 6-inch- (152-mm-) deep dispensers. Dispensers 4 inches (102 mm) in depth accommodate C-fold or multifold towels. C-fold towels are the highest quality and softest folded towels; single-fold towels are the least expensive.

          - Minimum Capacity: [400 C-fold or 525 multifold towels] [600 C-fold or 800 multifold towels] [400 single-fold towels] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

Retain "Refill Indicator" Subparagraph below if required.

          - Refill Indicator: Pierced slots at sides or front.

Roll dispensers extend farther from the wall than folded dispensers.

        - Paper Towel (Roll) Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Lever-actuated mechanism permits controlled delivery of paper rolls in preset lengths per stroke.

          - Mounting: [Recessed] [Semirecessed] [Surface mounted].

          - Minimum Capacity: [8-inch- (203-mm-) wide, 800-foot- (244-m-) long roll] <Insert capacity>.

          - Material and Finish: [Stainless steel, ASTM A480/A480M No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

          - Lockset: Tumbler type.

        - Liquid-Soap Dispenser <Insert drawing designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

          - Manufacturers: Subject to compliance with requirements, provide products by one of the following:

            - AJW Architectural Products.

            - American Specialties, Inc.

            - Bobrick Washroom Equipment, Inc.

            - Bradley Corporation.

            - Brey-Krause Manufacturing Co.

            - GAMCO Specialty Accessories; a division of Bobrick.

            - Tubular Specialties Manufacturing, Inc.

            - <Insert manufacturer's name>.

          - Description: Designed for dispensing soap in [liquid or lotion] [lather] form.

          - Mounting: [Deck mounted on vanity] [Deck mounted on lavatory] [Horizontally oriented, recessed] [Horizontally oriented, surface mounted] [Vertically oriented, surface mounted].

          - Capacity: <Insert oz. (mL)>.

          - Materials: <Insert requirements for valve and reservoir materials and finishes>.

Retain "Lockset" and "Refill Indicator" subparagraphs below if required.

          - Lockset: Tumbler type.

          - Refill Indicator: Window type.

      - MATERIALS

Retain this article if not naming manufacturers' products or if material descriptions are used in part to determine the selection of products.

        - Stainless Steel: ASTM A240/A240M or ASTM A666, Type 304, 0.031-inch (0.8-mm) minimum nominal thickness unless otherwise indicated.

        - Brass: ASTM B19, flat products; ASTM B16/B16M, rods, shapes, forgings, and flat products with finished edges; or ASTM B30, castings.

        - Steel Sheet: ASTM A1008/A1008M, Designation CS (cold rolled, commercial steel), 0.036-inch (0.9-mm) minimum nominal thickness.

        - Galvanized-Steel Sheet: ASTM A653/A653M, with G60 (Z180) hot-dip zinc coating.

        - Galvanized-Steel Mounting Devices: ASTM A153/A153M, hot-dip galvanized after fabrication.

        - Fasteners: Screws, bolts, and other devices of same material as accessory unit and tamper-and-theft resistant where exposed, and of galvanized steel where concealed.

Retain "Chrome Plating" Paragraph below for chrome finish. Brass, steel, and cast-zinc alloy (zamac) are available with chrome finish.

        - Chrome Plating: ASTM B456, Service Condition Number SC 2 (moderate service).

Revise "Mirrors" Paragraph below if tempered or safety glass is required; ASTM C 1503 does not cover tempered or safety glass mirrors. Note that reflected images with tempered or safety glass may be distorted.

        - Mirrors: ASTM C1503, Mirror Glazing Quality, clear-glass mirrors, nominal 6.0 mm thick.

      - FABRICATION

        - General: Fabricate units with tight seams and joints, and exposed edges rolled. Hang doors and access panels with full-length, continuous hinges. Equip units for concealed anchorage and with corrosion-resistant backing plates.

        - Keys: Provide universal keys for internal access to accessories for servicing and resupplying. Provide minimum of [six] <Insert number> keys to Owner's representative.

- EXECUTION

      - INSTALLATION

        - Install accessories in accordance with manufacturers' written instructions, using fasteners appropriate to substrate indicated and recommended by unit manufacturer. Install units level, plumb, and firmly anchored in locations and at heights indicated.

        - Grab Bars: Install to withstand a downward load of at least 250 lbf (1112 N), when tested in accordance with ASTM F446.

      - ADJUSTING AND CLEANING

        - Adjust accessories for unencumbered, smooth operation. Replace damaged or defective items.

        - Remove temporary labels and protective coatings.

        - Clean and polish exposed surfaces in accordance with manufacturer's written instructions.

END OF SECTION 102800